VOIS for Tech University Engagement Program

Innovation Marathon 2025-2026

Team No.- 46

Project Title:  Real-Time ECG Monitoring, Anomaly Detection and Stress Detection System Using ESP32 Using Wearable Biosensors.

Problem statement :  This project develops a low-cost, biomedical sensor system (ECG, pulse, temp, SpO2) using ESP32 for real-time stress and cardiac monitoring for remote healthcare access.

Proposed solution:

Build a compact wearable/patch or small chest/arm module centered on an ESP32. Use an ECG front-end (AD8232/ADS1292R) for ECG, MAX30102 for pulse/SpO₂ (PPG), and DS18B20 or MLX90614 for temperature. Do preprocessing and basic analytics on the ESP32 (filtering, QRS/pulse detection, HR/HRV features). Send summarized data and alerts over MQTT/TLS (Wi-Fi) to a cloud backend (InfluxDB/Grafana or Firebase) for visualization and long-term analytics. Run a lightweight classifier either on-device (TensorFlow Lite Micro) or server-side to infer stress level from HRV + PPG + temperature features. Provide a mobile/web dashboard for clinicians and caregivers with real-time stream, historical trends, and push/SMS alerts.

Technology to be used:

# Hardware

- ESP32-WROOM (Wi-Fi + BLE) — MCU and comms.

- ECG front-end: AD8232 (cheap, single-lead, suited for prototypes) OR ADS1292R (more clinical, 24-bit ADC, higher quality).

- PPG/SpO₂ sensor: MAX30102 or MAX30105 (I²C).

- Temperature: DS18B20 (contact, simple) or MLX90614 (non-contact IR).

- Power: LiPo battery (500–2000 mAh), TP4056 charger, boost/buck if needed.

- Aux: electrodes (Ag/AgCl), resistor pads, connectors, small PCB, enclosure.

ML / training: Python (scikit-learn, XGBoost, PyTorch/TensorFlow)